RC Time Constant Quiz Answer Key

Key Equations

1. Which of the following is the most accurate graph for the relationship of charge being dissipated out of a capacitor versus time?

A. 	B.

C. 	D.

Image Source: http://www.bbc.co.uk/bitesize/higher/physics/mech_matt/analyse_motion/revision/1/

2. Explain why the time it would take to discharge half of the capacitor’s charge is NOT equal to half of the total time.

This is not a linear relationship; it is logarithmic. More charge is lost at the beginning of the discharging process.

3. A capacitor is used to store charge for a disposable camera flash.  One second after the flash is operated, the charge left in the capacitor is 5 mC.  If the capacitance is 0.08 µF and the resistance of the light bulb is 25 Ω, what is the initial charge right before the flash is operated?

Q0 = 0.003 C

4. Using all of the information from question 4, what percentage of the original charge was left after 1 second?

60%

5. Using all of the information from question 4, what is the current after 1.2 seconds if the initial current right as the flash is operated is 2 A?

I(1.2s) = 2A (Q/Q0) = 1.1 A

6. Find the time constant of a capacitor if after 5 seconds of discharge, it had 1/4 as much as the initial charge.

T = RC = 3.61 seconds

7. What are the conditions for finding the RC time constant of a particular capacitor?

RC =t, or The elapsed time must be equal to the product of the resistance and the capacitance

Figure 1: A circuit with a capacitor, a resistor, and a switch.  The capacitor has capacitance 40 µF, the resistor has resistance 90Ω.  Use this figure to answer questions 6-9.

8. Find the RC time constant for the circuit in figure 1.

RC = (90Ω)(40µF) = 36 sec

9. Say you flip the switch closed (Figure 1).  After how much time will the capacitor in figure 1 have half of its initial charge remaining?

24.95 seconds

10. At what time will the current be one fifth (20%) of what it was when you initially flipped the switch? (Use Figure 1)

t = 57.94 seconds