Resistors in Circuits Quiz Answer Key

Key Equations

1. True or False:  The total resistance in a circuit is equal to the sum of all of the resistors. ______False______

2. Choose the best answer.

a) In a series circuit, the current is the same throughout the loop

b) In a parallel circuit, the current is the same throughout the circuit

c) All resistors in a circuit experience the same voltage drop

d) All of the above are true

3. Fill in the Blanks with either “Series” or “Parallel.”

When calculating the total resistance of a circuit, it is easiest to first simplify all of the resistors that are wired in _____Parallel________, and then add all resistances that are wired in ______Series_______.

http://happygj.wikispaces.com/file/view/Simple+Circuit+for+AT+-+EEX+4763.pdf

Figure 1: A series circuit with a battery between points A and F and a lightbulb between points J and E.  Use this figure to answer questions 4 and 5.

4. Consider the circuit in figure 1.  To what two points would you connect a voltmeter to measure the full voltage of the battery?

Connect to A and F

5. Consider the circuit in figure 1, and assume the lightbulb between points E and J has a resistance of 10Ω.  What could you do to this circuit create a total resistance of 3/5 Ω?

Add two resistors in parallel; one with resistance 6Ω and one with resistance 3Ω

6. Draw the schematic diagram for the following circuit:  One battery and one light bulb wired in series, with a parallel branch of two resistors, and a parallel branch off of one resistor with a third resistor.

Image Source: http://www.docstoc.com/docs/5630564/Mixed-Circuit-Diagrams-Hints-Simplify-the-circuits-Try-and

Figure 2: A circuit with both series and parallel elements.  The resistor at the top, called R1, has a voltage drop of 8V and a resistance of 0.8Ω.  The middle resistor, called R2, has a voltage drop of 12V and a Resistance of 3Ω.  The outermost resistor on the right, called R3, has a resistance of 2Ω.  The battery is all the way on the left.  Use this image to answer questions 7 through 10.

7. What is the total resistance of this circuit?

Rtot = 2Ω

7. Using the information from figure 2, what is the voltage across R3, the outermost resistor?

V3 = V2 = 12V

8. Using the information from figure 2, what is the total current running out of the voltage source?

I = 10 amps

9. Using the information from figure 2, what is the current through each of the three resistors?

I1 = 10 amps

I2 = 4 amps

I3 = 6 amps

10. Using the information given in the figure, what is the power dissipated by each of the three resistors?

P1 = 80 Watts

P2 = 48 Watts

P3 = 72 Watts