Resistors in Parallel Quiz Answer Key

Key Equations

1. True or False:  Adding more resistors in parallel will increase the total resistance. ______True______

2. If two identical 10-Ω resistors are connected in a parallel circuit, the total resistance of the circuit will be:

a) 20Ω

b) 10Ω

c) 5Ω

d) 1Ω

Image Source: http://www.ck12.org/concept/Resistors-in-Parallel/?ref=%2Fconcept%2FResistors-in-Parallel%2F

Figure 1: A circuit diagram with a 20-volt voltage source, a 90 Ω resistor called R1, and a 10-Ω resistor, R2.  Use this image to answer questions 3 and 4.

3. Consider figure 1.  What is the total resistance of the circuit, discluding the resistance in the wire?

Rtotal = 9Ω

4. Consider figure 1, and use your answer from question 3.  What is the current flowing out of the power supply?

I = 2.2 Amps

Image Source: http://rborovikov.blogspot.com/2011/03/assignment-c288.html

Figure 2: A circuit diagram with a voltage source and two resistors in parallel, R1 and R2.  Use this image to answer questions 5 through 7.

5. Consider Figure 2, and assume the voltage source has a voltage of 120 Volts.  What is the total resistance if there is a current flow of 5 Amps through R1 and a flow of 2 Amps in R2?

R1 = 24Ω, R2 = 60Ω, Rtotal = 17.14 Ω

6. Consider figure 2, and assume the voltage source has a voltage of 6 Volts.  If the two resistors are equal, what are the resistances of R1 and R2 such that there is a current flow of 8 Amps from the voltage source?

R1 = R2 = 8/3 Ω = 2.67 Ω

7. Consider figure 2. In order to raise the current running through the entire circuit, woud you add a resistor in series or in parallel?

Parallel

8. Draw a schematic circuit diagram of a 120-volt power source wired in parallel with 2 resistors, one with resistance R1 = 20Ω, one with resistance R2 = 22Ω, and one with resistance R3 = 14Ω.  How much current is flowing out of the voltage source?

Rtotal = 5.99 Ω

I = 20 Amps

9. Consider question 8.  How much power is being dissipated by each of the three resistors?

P1 = 720 Watts

P2 = 654.5 Watts

P3 = 1028.6 Watts

10. Consider your answers to question 9.  What is the relationship between the resistance of each resistor and the power dissipated by each?  Discuss why this is.

The larger resistor receives less current flow, thus producing less power, because the higher resistance will better prevent a flow.